Module 8: Network Access Basic routing and Advance Routing Concept, Switching Concept

- Explain Switch

A switch is a networking device used to connect multiple devices such as computers, printers, and servers in a Local Area Network (LAN). It transfers data efficiently by using MAC addresses and sends data only to the intended device instead of broadcasting it to all devices. This helps improve network speed, performance, and security.

- Explain Switch Boot Sequence

The switch boot sequence is the process a network switch follows when it is powered on. First, the switch performs a Power-On Self-Test (POST) to check hardware components such as memory and CPU. Then, it loads the boot loader program, which finds and loads the operating system from flash memory into RAM. After the operating system starts, the switch loads the saved configuration file and becomes ready for network communication.

- Explain Three Methods to access Switch Command Line Interface.

- Console Port Method – This method uses a console cable to connect a computer directly to the switch. It is mainly used for initial setup and troubleshooting because it does not require network connectivity.

Telnet Method – Telnet allows users to access and configure the switch remotely through a network connection. It is simple to use, but it is less secure because usernames, passwords, and data are sent in plain text.

SSH (Secure Shell) Method – SSH is a secure way to remotely access the switch over a network. It encrypts all communication between the user and the switch, making it safer than Telnet for managing network devices.

4. Explain and Configuring the Cisco Internet Operating System.

The Cisco Internetwork Operating System (IOS) is the operating system used in Cisco networking devices such as routers and switches. It provides the interface for configuring, managing, and monitoring network devices through the Command Line Interface (CLI).

Basic Cisco IOS Configuration Commands

- Access Privileged Mode

Switch> enable

- Enter Global Configuration Mode

Switch# configure terminal

- Set Hostname

Switch(config)# hostname Switch1

- Set Password for Console Access

Switch1(config)#line console 0
Switch1(config-line)# password cisco
Switch1(config-line)# login

- Set Enable Secret Password

Switch1(config)# enable secret class

- Configure IP Address

Switch1(config)# interface vlan 1
Switch1(config-if)# ip address 192.168.1.1 255.255.255.0
Switch1(config-if)# no shutdown

- Save Configuration

Switch1# copy running-config startup-config

5.Explain Switch Port.

A switch port is a physical or logical interface on a switch where network devices like computers, printers, or other switches are connected. It is used to send and receive data between devices within a network.

Each switch port can be configured to control speed, security, VLANs, and communication settings. There are two main types of switch ports: access ports (connect end devices in a single VLAN) and trunk ports (carry traffic for multiple VLANs between switches).

MCQ

1. 4-R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two)

A. R1

B. R2

C. R3

D. R4 E. None of the routers will exchange routing information

Any

2. 3-enable secret hashed using the---------- algorithm.

A. MD5

B. AH

C. PSK

D. ESP

E. WPA2

Ans=(A.MD5)

4- An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A. R1 is an Area Border Router.

B. R1 is a backup designated router.

C. Router 2.2.2.2 is an Area Border Router.

D. Router 2.2.2.2 is a backup designated router.

Ans=(D. Router 2.2.2.2 is a backup designated router.)

5- Which command is used to view the neighbor discovery table on a PC?

A. show ipv6 neighbor

B. show ipv6 neighbors

C. netsh interface ipv6 show neighbor

D. netsh interface ipv6 show neighbors

Ans=(C. netsh interface ipv6 show neighbor)

6- What type of variable is being shown? Routers = [R1,R2,R3]

A. List

B. Dictionary

C. Simple

D. Unsigned integers

Ans=( A. List)

7- Identify the fields in an IPv4 header. (Choose three)

A. Host component

B. Time to Live

C. Source address

D. Destination address

Ans=(B. Time To Live
            C. Source address
            D. Destination address)