Module -7: Network fundamental –

1- Which of the following messages in the DHCP process are broadcasted? (Choose two)

A. Request

B. Offer

C. Discover

D. Acknowledge

Ans=(A. Request, C. Discover)

2- Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any

B. permit tcp any any eq 80

C. permit tcp any eq 80

D. permit any any eq tcp

Ans=( B. permit tcp any any eq 80)

3-Explain Network Topologies?

There are several types of network topologies used in practice.

- Bus topology

- Star topology

- Ring topology

- Mesh topology

- Tree topology

- Hybrid topology

Bus Topology:- In a bus topology, all devices are connected to a single backbone cable, making it simple and low-cost, but a failure in the main cable can disrupt the entire network.

Star Topology:- A star topology connects all devices to a central hub or switch, which makes it easy to manage and troubleshoot; however, if the central device fails, the whole network stops.

Ring Topology:- In a ring topology, devices are connected in a circular manner, and data travels in one direction, reducing data collisions, but failure of one node can affect the entire system.

Mesh Topology:- A mesh topology connects each device to multiple other devices, providing high reliability and redundancy, though it is expensive and complex to install.

Tree Topology:- A tree topology is a hierarchical structure that combines characteristics of bus and star topologies, commonly used in large organizations for better scalability.

Hybrid Topology:- Hybrid topology is a combination of two or more topologies, offering flexibility and efficiency in large and complex networks.

4-Explain TCP/IP Networking Model?

Application Layer is the top layer and is responsible for providing network services to users and applications. It includes protocols such as HTTP, FTP, and SMTP, which allow activities like web browsing, file transfer, and email communication. This layer interacts directly with user applications.

Transport Layer ensures reliable data transmission between devices. It uses protocols like TCP, which provides reliable and connection-oriented communication, and UDP, which is faster but does not guarantee delivery. This layer is responsible for error checking, data flow control, and segmentation.

Internet Layer is responsible for addressing and routing data packets across networks. It uses the IP to assign unique addresses to devices and determine the best path for data transmission. This layer ensures that data reaches the correct destination.

Network Access Layer (also called the Link Layer) handles the physical transmission of data over the network medium such as cables or wireless signals. It includes technologies like Ethernet and Wi-Fi and is responsible for framing, hardware addressing, and error detection.

5-Explain LAN and WAN Network?

LAN (Local Area Network) is a network that connects computers and devices within a small geographical area, such as a home, school, office, or building. It is used to share resources like files, printers, and internet connections. LANs are usually fast, secure, and inexpensive because they cover a limited area and are often privately owned. Common technologies used in LANs include Ethernet cables and Wi-Fi.

WAN (Wide Area Network) connects devices over a large geographical area, such as cities, countries, or even the entire world. The best example of a WAN is the Internet, which links millions of networks globally. WANs are generally slower and more expensive than LANs because they require public infrastructure like leased lines, satellites, or telecom networks. They are used by organizations to connect branch offices located in different locations.

6-Explain Operation of Switch?

Operation of a Switch (Point-wise)

- A switch is a networking device used to connect multiple devices in a LAN.

Working Steps:

- Frame Reception

  - Switch receives a data frame from a device.

- Read Source MAC Address

  - It checks the source MAC address.

  - Stores it in the MAC address table (CAM table) with port number.

- Check Destination MAC Address

  - Switch looks for the destination MAC in its table.

- Forwarding Decision

  - If found → Sends frame only to the correct port.

  - If not found → Floods frame to all ports (except incoming port).

7-Describe the purpose and functions of various network devices?

1. Hub

- Purpose: Connect multiple devices in a network

- Function:

  - Sends data to all devices (broadcast)

  - Works at Physical Layer (Layer 1)

- Note: Outdated, replaced by switches

2. Switch

- Purpose: Connect devices in a LAN efficiently

- Function:

  - Uses MAC addresses to send data to the correct device

  - Reduces traffic and collisions

  - Works at Data Link Layer (Layer 2)

3. Router

- Purpose: Connect different networks (e.g., LAN to Internet)

- Function:

  - Uses IP addresses to route data

  - Finds best path for data transmission

  - Works at Network Layer (Layer 3)

4. Modem

- Purpose: Connect network to Internet via ISP

- Function:

  - Converts digital signals ↔ analog signals

  - Enables internet access

5. Access Point (AP)

- Purpose: Provide wireless connectivity

- Function:

  - Connects Wi-Fi devices to wired network

  - Extends network coverage

6. Bridge

- Purpose: Connect two LAN segments

- Function:

  - Filters traffic using MAC addresses

  - Reduces unnecessary data transfer

7. Repeater

- Purpose: Extend network range

- Function:

  - Regenerates and boosts signals

  - Prevents signal loss

8. Gateway

- Purpose: Connect different types of networks

- Function:

  - Translates protocols

  - Acts as entry/exit point of network

8-Make list of the appropriate media, cables, ports, and connectors to 8-connect switches to other?

1. Twisted Pair Cable (Ethernet Cable)

- Media: Copper cable

- Types: UTP (Unshielded), STP (Shielded)

- Connector: RJ-45

- Port: Ethernet port on switch (Fast Ethernet / Gigabit Ethernet)

- Use:

  - Switch ↔ Computer

  - Switch ↔ Router

  - Switch ↔ Switch (modern switches use auto MDI-X)

2. Straight-Through Cable

- Connector: RJ-45

- Use:

  - Switch ↔ PC

  - Switch ↔ Router

3. Crossover Cable

- Connector: RJ-45

- Use:

  - Switch ↔ Switch (older devices)

  - Switch ↔ Hub

4. Fiber Optic Cable

- Media: Optical fiber

- Types: Single-mode, Multi-mode

- Connectors: SC, ST, LC

- Port: SFP (Small Form-factor Pluggable) port on switch

- Use:

  - Switch ↔ Switch (long distance, high speed)

5. Console Cable (Rollover Cable)

- Connector: RJ-45 to Serial/USB

- Port: Console port on switch

- Use:

  - For configuration (PC ↔ Switch)

6. Coaxial Cable (Rare)

- Connector: BNC

- Use:

  - Old networks (not used in modern switches)

9-Define Network devices and hosts?

1. Network Devices

- Definition:
Network devices are hardware components used to connect, manage, control, and transmit data between different devices in a network.

- Functions:

  - Direct data traffic

  - Connect different networks

  - Ensure communication between devices

- Examples:

  - Router

  - Switch

  - Hub

  - Modem

  - Access Point

2. Hosts

- Definition:
Hosts are end devices in a network that send, receive, or use data.

- Functions:

  - Act as source or destination of data

  - Run applications and services

- Examples:

  - Computer

  - Laptop

  - Smartphone

  - Printer

  - Server